HISPL HYDRAULIC CYLINDER COSTING SYSTEM

MASTER TABLES — DEVELOPER REFERENCE

Extracted from Trunion Included (1).xlsx | Seal master excluded as requested

# 1. Master Sheets in the Supplied Workbook

| Master | Purpose |
| --- | --- |
| Material Master | Material grade, density and material rate |
| Machine Rate Master | Machine hourly rates and machine/process mapping |
| Machine Time Master | Dimension/range based machine-time standards |
| Process Rate Master | Area/weight process rates, turning/honing cards and welding constants |
| Bought Out & Seal Kit Master | Bought-out items and seal kit; only bought-out portion is reproduced here |

# 2. Material Master — Current Values

| Material Code | Material Name / Grade | Density (g/cm3) | Rate (Rs./kg) | Remarks |
| --- | --- | --- | --- | --- |
| EN8 | EN8 Carbon Steel | 7.85 | 68.0 | Tubes, general turning parts |
| EN19 | EN19 Alloy Steel | 7.85 | 92.0 | Piston Rod - heat treatable |
| EN24 | EN24 Alloy Steel | 7.85 | 118.0 | High strength rods / eyes |
| C45 | C45 Carbon Steel | 7.85 | 66.0 | Cap/Head End Covers, Gland, Flange |
| ST52 | ST52 Seamless Tube | 7.85 | 78.0 | Cylinder Tube raw material |
| SS-410 | SS 410 Stainless | 7.7 | 210.0 | Corrosion resistant rods |
| BR-SAE660 | Bronze SAE 660 | 8.9 | 520.0 | Cushion Bush / Bearing Bush |
| EN353 | EN353 Case Hardening Steel | 7.85 | 96.0 | Pins / Bushes |
| PLATE-IS2062 | IS2062 Plate | 7.85 | 62.0 | Rear Eye / Rod Eye / Piston blanks |
| MS |  |  |  |  |

# 3. Machine Rate Master — Current Values

| Machine | Hourly Rate (Rs./Hr) | Process(es) |
| --- | --- | --- |
| CNC Lathe | 550.0 | Finish Turning (rate overridden by Turning Rate Card below); generic Turning on CEC/HEC/Flange/Piston |
| Center Lathe | 500.0 | Boring; Rough Turning (rate overridden by Turning Rate Card below) |
| Milling Machine | 350.0 | Milling |
| Drilling Machine | 250.0 | Drilling |
| Honing Machine | 450.0 | Rough Honing and Finished Honing (utilisation reference; cost via Honing Rate Card) |
| Grinding Machine | 400.0 | Grinding (utilisation reference; cost via Process Rate Master Rs./cm2) |
| Cutting Machine | 350.0 | Cutting - universal flat charge |

# 4. Machine Time Master — Current Values

## 4.1 Cutting Time

| SECTION 2 - CUTTING TIME (Hrs) ¦ Inputs: OD, Length | None | None | None | None | None |
| --- | --- | --- | --- | --- | --- |
| Bin Start | OD (mm) | Up to 500 | 501-1000 | 1001-2000 | 2001-3000 |
|  | Length/Area Bin Start (helper) | 0.0 | 501.0 | 1001.0 | 2001.0 |
| 0.0 | Up to 80 | 0.08 | 0.1 | 0.15 | 0.2 |
| 81.0 | 81-150 | 0.1 | 0.15 | 0.2 | 0.3 |
| 151.0 | 151-250 | 0.15 | 0.2 | 0.3 | 0.4 |

## 4.2 Rough Turning Time

| SECTION 3 - ROUGH TURNING TIME (Hrs) ¦ Inputs: Finished OD, Length. Stock Removal = Raw OD - Finished OD | None | None | None | None | None |
| --- | --- | --- | --- | --- | --- |
| Bin Start | Finished OD (mm) | Up to 500 | 501-1000 | 1001-2000 | 2001-3000 |
|  | Length/Area Bin Start (helper) | 0.0 | 501.0 | 1001.0 | 2001.0 |
| 0.0 | Up to 80 | 0.3 | 0.6 | 1.2 | 1.8 |
| 81.0 | 81-150 | 0.5 | 0.9 | 1.7 | 2.5 |

## 4.3 Stock Removal Correction Factor

| 251.0 | Above 250 | 1.2 |
| --- | --- | --- |
| STOCK REMOVAL CORRECTION FACTOR (applied to Rough Turning Time) |  |  |
| Bin Start (helper) | Stock Removal (mm) | Factor |
| 0.0 | <= 2 mm | 1.0 |

## 4.5 Boring Time

| 10.0001 | > 10 mm | 1.6 | None | None | None |
| --- | --- | --- | --- | --- | --- |
| SECTION 4 - Finish Turning Time = Rough Turning Time (looked up the same way) x 0.70 (applied directly in component sheets) |  |  |  |  |  |
| SECTION 5 - BORING TIME (Hrs) ¦ Inputs: Finished ID, Bore Length |  |  |  |  |  |
| Bin Start | ID (mm) | Up to 250 | 251-500 | 501-1000 | 1001-2000 |

## 4.6 Honing Time

| 0.0 | Up to 80 | 0.25 | 0.4 | 0.7 |
| --- | --- | --- | --- | --- |
| 81.0 | 81-150 | 0.4 | 0.7 | 1.2 |
| 151.0 | 151-250 | 0.7 | 1.2 | 2.2 |
| 251.0 | Above 250 | 1.2 | 2.0 | 3.5 |
| SECTION 6 - HONING TIME (Hrs, utilisation reference) ¦ Cost is via Honing Rate Card (Process Rate Master), NOT this table |  |  |  |  |

## 4.7 Grinding Time

| None | Length/Area Bin Start (helper) | 0.0 | 501.0 | 1001.0 |
| --- | --- | --- | --- | --- |
| 0.0 | Up to 80 | 0.3 | 0.6 | 1.2 |
| 81.0 | 81-150 | 0.5 | 0.9 | 1.6 |
| 151.0 | 151-250 | 0.8 | 1.4 | 2.4 |
| 251.0 | Above 250 | 1.2 | 2.0 | 3.5 |

## 4.8 Milling Time

| Grinding COST is charged via Process Rate Master (Rs./cm2). This table is for machine hour / utilisation tracking on the Dashboard. | None | None |
| --- | --- | --- |
| Bin Start | OD (mm) | Up to 500 |
|  | Length/Area Bin Start (helper) | 0.0 |
| 0.0 | Up to 80 | 0.3 |
| 81.0 | 81-150 | 0.5 |
| 151.0 | 151-250 | 0.8 |

## 4.9 Drilling Time

| SECTION 8 - MILLING TIME (Hrs) ¦ Input: Machined Area (mm2) = Width x Length | None | None |
| --- | --- | --- |
| Bin Start (helper) | Machined Area (mm2) | Time (Hr) |
| 0.0 | Up to 10,000 | 0.3 |
| 10001.0 | 10,001-25,000 | 0.6 |
| 25001.0 | 25,001-50,000 | 1.0 |

## 4.10 Profile Cutting Time

| SECTION 9 - DRILLING TIME (Hrs per hole) ¦ Total Time = Time/Hole x No. of Holes | None | None |
| --- | --- | --- |
| Bin Start (helper) | Hole Dia (mm) | Time/Hole (Hr) |
| 0.0 | Up to 10 | 0.03 |
| 11.0 | 11-20 | 0.05 |
| 21.0 | 21-30 | 0.08 |

## 4.11 Surface Area Master

| SECTION 10 - PROFILE CUTTING TIME (Hrs, utilisation reference) ¦ Input: Weight (kg) | None |
| --- | --- |
| Profile Cutting COST is charged via Process Rate Master (Rs./kg). This table is for utilisation tracking. |  |

## 4.4 Finish Turning Time

Current workbook rule: Finish Turning Time = Rough Turning Time × 0.70. The component sheets apply this directly.

# 5. Process Rate Master — Current Values

| Process | Rate | Unit | Remarks |
| --- | --- | --- | --- |
| Process | Rate | Unit | Remarks |
| Heat Treatment | 12.0 | Rs./kg |  |
| Induction Hardening | 0.45 | Rs./cm2 |  |
| Grinding | 0.4 | Rs./cm2 |  |
| Polishing | 0.2 | Rs./cm2 |  |
| Painting | 0.2 | Rs./cm2 |  |
| Chrome Plating | 0.6 | Rs./cm2 |  |
| Dechrome Plating | 0.35 | Rs./cm2 |  |
| Profile Cutting | 1.25 | Rs./kg |  |
| Packing - Loose | 5.0 | Rs./kg |  |

## 5.1 Turning Rate Card

| None | None | None | None |
| --- | --- | --- | --- |
| TURNING RATE CARD (Conventional / CNC Lathe - by Finished Diameter) |  |  |  |
| Overrides the flat machine rate for Rough Turning and Finish Turning specifically, in Tube and Piston Rod sheets. |  |  |  |

## 5.2 Honing Rate Card

| 0.0 | Up to 100 mm | 300.0 | 400.0 |
| --- | --- | --- | --- |
| 101.0 | 101-250 mm | 550.0 | 550.0 |
| 251.0 | Above 250 mm | 700.0 | 700.0 |
| HONING RATE CARD (Rough Honing & Finished Honing) |  |  |  |
| Cost = Internal Surface Area (cm2) x Rate. Threshold applies to BOTH Length (Stroke) AND ID together. |  |  |  |

## 5.3 Welding Constants in Supplied Workbook

| Parameter | Value |
| --- | --- |
| Length / Stroke Threshold (mm) | 4000.0 |
| ID Threshold (mm) | 100.0 |
| WELDING CONSTANTS (HISPL FORMULA) |  |
| Labour Rate (Rs./Hr) | 375.0 |
| Wire Cost (Rs./kg) | 360.0 |
| Deposition Rate (kg/Hr) | 0.8 |
| Welding Speed (mm/Hr) | 3600.0 |
| Dia Threshold for Bead Count (mm) | 250.0 |
| Beads if Dia <= Threshold | 5.0 |
| Beads if Dia > Threshold | 8.0 |

Note: this supplied workbook still contains the original welding-constants block. The later approved HISPL instruction to use ₹14 per inch per bead should be applied as a subsequent controlled update if it is not yet reflected in the workbook.

# 6. Bought-Out Master — Excluding Seal

| Item Description | Category | Include? | Rate (Rs.) | Qty | Cost (Rs.) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Bearing | Bought Out | Yes | 180.0 | 2.0 | =IF(C5="Yes",D5*E5,0) |  |
|  | Bought Out | No | 0.0 | 1.0 | =IF(C6="Yes",D6*E6,0) |  |
| Check Valve | Bought Out | No | 650.0 | 1.0 | =IF(C7="Yes",D7*E7,0) |  |
| Transducer | Bought Out | No | 4500.0 | 1.0 | =IF(C8="Yes",D8*E8,0) | Position sensor / LVDT, if applicable |
| Bellows | Bought Out | No | 850.0 | 1.0 | =IF(C9="Yes",D9*E9,0) | Rod protection bellow, if applicable |
| Other Component 1 | Bought Out | No | 0.0 | 1.0 | =IF(C10="Yes",D10*E10,0) | Manual entry - rename as needed |
| Other Component 2 | Bought Out | No | 0.0 | 1.0 | =IF(C11="Yes",D11*E11,0) | Manual entry - rename as needed |
| Other Component 3 | Bought Out | No | 0.0 | 1.0 | =IF(C12="Yes",D12*E12,0) | Manual entry - rename as needed |
| Other Component 4 | Bought Out | No | 0.0 | 1.0 | =IF(C13="Yes",D13*E13,0) | Manual entry - rename as needed |
| Other Component 5 | Bought Out | No | 0.0 | 1.0 | =IF(C14="Yes",D14*E14,0) | Manual entry - rename as needed |

The Seal Kit row and all seal-related data are intentionally excluded. Other Component rows remain manual placeholders.

# 7. Current Primary Inquiry Inputs

| Input | Current Workbook Value |
| --- | --- |
| Inquiry No. | INQ-0001 |
| Inquiry Date | =TODAY() |
| Customer Name | Sample Customer Pvt Ltd |
| Customer Location | Bengaluru |
| Cylinder Name / Application Department | Excavator Boom Cylinder |
| Bore (mm) | 100.0 |
| Rod Diameter (mm) | 56.0 |
| Stroke (mm) | 800.0 |
| Mounting Type | Clevis |
| Working Pressure (bar) | 250.0 |

# 8. Future Geometry Masters Required

The supplied workbook does not currently contain dedicated geometry masters for the following. These should be created as controlled engineering reference tables and populated from HISPL historical drawings/data or approved engineering standards.

| Future Master | Required fields |
| --- | --- |
| Geometry Master | Component, Bore From/To, Rod From/To, Stroke From/To, Tube OD From/To, Mounting, Standard, Min, Max, Override, Basis, Source, Confidence |
| Tube Geometry Master | Bore, Tube OD, Boring Allowance, OD Turning Allowance, Raw ID, Raw OD, Length Basis |
| Piston Rod Geometry Master | Rod Diameter, Bore Band, Raw OD Allowance, Finish OD, Length, Grinding/Plating/Polishing Length |
| Piston Geometry Master | Bore Band, Piston OD, Width, Rod Connection, Wear Ring Arrangement, Source |
| CEC Geometry Master | Bore Band, Tube OD Band, CEC OD, Thickness, Length, Port, Bolt Size, Bolt Circle, Bolt Qty |
| HEC Geometry Master | Bore Band, Tube OD Band, HEC OD, Thickness, Length, Port, Bolt Size, Bolt Circle, Bolt Qty |
| Gland Geometry Master | Bore, Rod Diameter, OD, ID, Length, Guide Length, Seal Groove Reference, Retaining Arrangement |
| Cushion Bush Geometry Master | Bore/Cushion input, OD, ID, Length, Material |
| Stop Tube Geometry Master | Bore Band, Required Y/N, ID, OD, Length Basis |
| Rod Eye Geometry Master | Rod Band, Pin Diameter, Eye OD, Eye Width, Boss Length, Hole Diameter, Weld Length |
| Rear Eye Geometry Master | Bore/Tube Band, Pin Diameter, Eye OD, Eye Width, Plate Thickness, Hole Diameter, Weld Length |
| Trunnion Geometry Master | Bore, Tube OD, Pressure, Trunnion OD, Pin Diameter, Thickness, Length, Bearing Width, Pin Length, Weld Geometry |
| Flange Geometry Master | Bore/Tube Band, OD, ID, Thickness, Bolt Circle, Bolt Qty, Hole Diameter, Weld Diameter |
| CEC Clevis Geometry Master | Bore Band, Lug Qty, Thickness, Width, Length, Pin Diameter, Hole Diameter, Weld Length |
| Foot Lug Geometry Master | Bore/Tube Band, Lug Qty, Thickness, Width, Length, Pin Diameter, Hole Diameter, Weld Length |
| Tie Rod Geometry Master | Bore Band, Qty, Diameter, Length, Thread Diameter, Thread Length |
| Pin Master | Pin Diameter, Clearance, Bearing Width, Material, Source |
| Port/Connection Master | Bore Band, Port Type, Port Size, Depth, Quantity, Source |

# 9. Geometry Calculation Rules

| Calculation | Formula / Basis |
| --- | --- |
| Finished Tube ID | Bore |
| Finished Tube OD | Tube OD input |
| Raw Tube ID | Bore + Tube Boring Allowance |
| Raw Tube OD | Tube OD + Tube OD Turning Allowance |
| Tube Volume | π/4 × (OD² − ID²) × Length |
| Solid Rod Volume | π/4 × OD² × Length |
| Ring/Bush Volume | π/4 × (OD² − ID²) × Length |
| Steel Weight | Volume (mm³) × 7.85 × 10⁻⁶ kg |
| Cylindrical Surface Area | π × Diameter × Length |
| Area conversion | mm² / 100 = cm² |
| Trunnion Thickness | (Trunnion OD − Pin Diameter) / 2 |
| Final Dimension | Manual Override if entered; otherwise Standard Geometry Value |

# 10. Machine-Time Geometry Keys

| Process | Inputs |
| --- | --- |
| Cutting | Raw length, OD/section |
| Rough Turning | Raw OD, finished OD, length, stock removal |
| Finish Turning | Finished OD/ID, length |
| Boring | Finished ID, bore length |
| Honing | ID, honing length, internal surface area |
| Grinding | OD, grinding length/area |
| Milling | Machined area = width × length |
| Drilling | Hole diameter, depth, number of holes |
| Profile Cutting | Component weight |
| Polishing | OD/length or surface area |
| Chrome Plating | OD/length or surface area |
| Induction Hardening | OD/length or surface area |
| Painting | Applicable external surface area |

# 11. Developer Control Rules

Use the supplied current master values as the source for the existing costing engine.

Do not overwrite validated costing formulas while adding geometry automation.

Do not invent missing geometry values. Missing engineering dimensions must be marked ENGINEERING INPUT REQUIRED.

Historical HISPL drawings/data and approved internal standards take priority over generic engineering assumptions.

Customer drawing dimensions must be supported through a manual override without changing the master.

Keep master data centralized; do not hard-code the same rate or geometry repeatedly in component sheets.

Changing Bore, Tube OD, Stroke or Rod Diameter must cascade to dependent geometry and costing.

Inactive mounting components must contribute zero cost.

Maintain source/basis and confidence for future geometry standards.

Do not use this costing geometry as final structural design approval unless separately checked and approved by HISPL engineering.

# 12. Implementation Sequence

Freeze the validated costing workbook.

Preserve Material, Machine Rate, Machine Time and Process Rate masters.

Create the Geometry Master and component-specific geometry lookup sections.

Load HISPL historical component dimensions into the geometry masters.

Link Inquiry Input to geometry keys.

Link geometry outputs to existing component sheets without replacing costing formulas.

Connect geometry to existing machine-time, welding, surface-area and weight calculations.

Test known cylinders against existing costing outputs.

Only then activate automatic geometry for new inquiries.

Store completed costings in Cylinder Database for future reference and Actual Cost Tracker comparison.

# 13. Source Boundary

Sections 2–7 reproduce the master data actually present in the supplied workbook. Section 8 defines the future geometry-master structure required for the next automation stage. No seal master data has been reproduced, as requested. No missing engineering dimension has been invented in this document.